#!/usr/bin/env python3

import time

import cv2

import numpy as np

from gpiozero import OutputDevice, PWMOutputDevice

from picamera2 import Picamera2

# ========== GPIO Setup ==========

# Left motor direction

in1 = OutputDevice(23)   # Pin 16

in2 = OutputDevice(24)   # Pin 18

ena = PWMOutputDevice(12)  # Pin 32 (PWM)

# Right motor direction

in3 = OutputDevice(17)  # Pin 11

in4 = OutputDevice(27)  # Pin 13

enb = PWMOutputDevice(13)  # Pin 33 (PWM)

# ========== Camera Setup ==========

camera = Picamera2()

camera_config = camera.create_preview_configuration(main={"size": (800, 480)})

camera.configure(camera_config)

camera.start()

print("Initializing camera...")

time.sleep(2)

cv2.namedWindow("Line Follower", cv2.WINDOW_NORMAL)

cv2.resizeWindow("Line Follower", 800, 600)

# ========== PID Parameters ==========

Kp = 0.25

Ki = 0.0005

Kd = 0.05

last_error = 0

integral = 0

# Speed settings (faster)

base_speed = 0.35

max_speed = 0.55

search_speed = 0.3

# Process flags

debug_mode = True

stop_motors_flag = False

# ========== Motor Control Functions ==========

def set_motor(left_dir, right_dir, left_speed, right_speed):

if left_dir == "forward":

in1.off()

in2.on()

else:

in1.on()

in2.off()

ena.value = abs(left_speed)

if right_dir == "forward":

in3.off()

in4.on()

else:

in3.on()

in4.off()

enb.value = abs(right_speed)

def stop_motors():

ena.off()

enb.off()

in1.off()

in2.off()

in3.off()

in4.off()

# ========== Image Processing ==========

def process_image(image):

height, width = image.shape[:2]

crop_top = int(height * 0.6)

crop_bottom = int(height * 0.8)

roi = image[crop_top:crop_bottom, 0:width]

hsv = cv2.cvtColor(roi, cv2.COLOR_BGR2HSV)

# Thresholds for black

lower_black = np.array([0, 0, 0])

upper_black = np.array([180, 255, 70])

# Thresholds for red (2 ranges in HSV)

lower_red1 = np.array([0, 100, 100])

upper_red1 = np.array([10, 255, 255])

lower_red2 = np.array([160, 100, 100])

upper_red2 = np.array([180, 255, 255])

mask_black = cv2.inRange(hsv, lower_black, upper_black)

mask_red1 = cv2.inRange(hsv, lower_red1, upper_red1)

mask_red2 = cv2.inRange(hsv, lower_red2, upper_red2)

mask_red = cv2.bitwise_or(mask_red1, mask_red2)

mask = cv2.bitwise_or(mask_black, mask_red)

kernel = np.ones((3, 3), np.uint8)

processed = cv2.erode(mask, kernel, iterations=1)

processed = cv2.dilate(processed, kernel, iterations=1)

contours, _ = cv2.findContours(processed, cv2.RETR_EXTERNAL, cv2.CHAIN_APPROX_SIMPLE)

debug_image = np.copy(image)

debug_roi = cv2.cvtColor(processed, cv2.COLOR_GRAY2BGR)

debug_image[crop_top:crop_bottom, 0:width] = debug_roi

cv2.rectangle(debug_image, (0, crop_top), (width, crop_bottom), (0, 255, 255), 2)

if contours:

largest = max(contours, key=cv2.contourArea)

if cv2.contourArea(largest) > 50:

M = cv2.moments(largest)

if M["m00"] != 0:

cx = int(M["m10"] / M["m00"])

cy = int(M["m01"] / M["m00"])

cv2.drawContours(debug_image[crop_top:crop_bottom], [largest], -1, (0, 255, 0), 2)

cv2.circle(debug_image[crop_top:crop_bottom], (cx, cy), 5, (0, 0, 255), -1)

cv2.line(debug_image, (width // 2, 0), (width // 2, height), (255, 0, 0), 1)

cv2.line(debug_image[crop_top:crop_bottom], (roi.shape[1] // 2, cy), (cx, cy), (255, 0, 0), 2)

cv2.putText(debug_image, f"Error: {cx - roi.shape[1] // 2}", (10, 30), cv2.FONT_HERSHEY_SIMPLEX, 0.7, (0, 255, 0), 2)

return cx - roi.shape[1] // 2, debug_image, True

cv2.putText(debug_image, "NO LINE DETECTED", (width // 2 - 100, height // 2), cv2.FONT_HERSHEY_SIMPLEX, 0.7, (0, 0, 255), 2)

return 100, debug_image, False

# ========== PID Control ==========

def pid_control(error, found_line):

global last_error, integral, stop_motors_flag

if stop_motors_flag:

stop_motors()

return

if not found_line:

set_motor("forward", "backward", search_speed, search_speed)

print("Searching for line...")

return

proportional = error

integral += error

derivative = error - last_error

integral = max(min(integral, 500), -500)

last_error = error

adjustment = Kp * proportional + Ki * integral + Kd * derivative

left_speed = base_speed - adjustment / 100

right_speed = base_speed + adjustment / 100

left_speed = max(min(left_speed, max_speed), -max_speed)

right_speed = max(min(right_speed, max_speed), -max_speed)

left_dir = "forward" if left_speed >= 0 else "backward"

right_dir = "forward" if right_speed >= 0 else "backward"

set_motor(left_dir, right_dir, abs(left_speed), abs(right_speed))

print(f"Left: {left_speed:.2f}, Right: {right_speed:.2f}, Error: {error}")

# ========== Main Loop ==========

try:

print("Line follower active. Press 'Q' to quit, 'P' to pause/resume.")

while True:

image = cv2.flip(camera.capture_array(), 1)

image = cv2.cvtColor(image, cv2.COLOR_RGBA2BGR)

error, debug_image, found_line = process_image(image)

pid_control(error, found_line)

if debug_mode:

cv2.imshow("Line Follower", debug_image)

key = cv2.waitKey(1) & 0xFF

if key == ord("q"):

break

elif key == ord("p"):

stop_motors_flag = not stop_motors_flag

print("Motors paused." if stop_motors_flag else "Motors resumed.")

except KeyboardInterrupt:

print("Stopped by user.")

finally:

stop_motors()

camera.stop()

cv2.destroyAllWindows()

print("Motors stopped. Camera released.")